Task2 – Wind Pressure Calculations

Methodology

The wind load acting on the guyed tubular monopole was determined using the equivalent static wind load procedure specified in ASCE 7. The methodology adopted is summarized in the following steps:

Step 1: Define the Structural Geometry

The monopole has an overall height of 100 ft (30.48 m) and is divided into five tubular sections. The lower four sections (M2) are each 20 ft (6.096 m) long with an outside diameter of 3 ft (0.9144 m), while the top section (M1) is 20 ft (6.096 m) long with an outside diameter of 2 ft (0.6096 m). All sections have a wall thickness of 5 mm.

Step 2: Select the Design Wind Parameters

The design wind parameters were selected according to ASCE 7 as follows:

- Basic wind speed, (V = 6.994) m/s

- Exposure Category = D

- Topographic factor, (Kzt= 1.0)

- Wind directionality factor, (Kd = 0.85)

- Gust effect factor, (G = 0.85)

Step 3: Determine the Evaluation Height

The wind pressure for each mast section was evaluated at its centroid. The centroid heights above the base are:

| Section | Centroid Height (ft) |
| --- | --- |
| Section 1 | 10 |
| Section 2 | 30 ft |
| Section 3 | 50 ft |
| Section 4 | 70 ft |
| Section 5 | 90 ft |

Step 4: Calculate the Velocity Pressure Exposure Coefficient

The velocity pressure exposure coefficient, (Kz)was calculated for each section using the ASCE 7 equation for Exposure Category D. Since wind speed varies with elevation, a separate value of (Kz) was determined for each centroid height.

Step 5: Calculate the Velocity Pressure

The velocity pressure at the centroid of each section was calculated using

qz = 0.613 * Kz *Kzt * Kd * V2

Where,
 (qz) is the velocity pressure (Pa).
(Kz) is the velocity pressure exposure coefficient, 
(Kzt) is the topographic factor,

(Kd) is the wind directionality factor,

(V) is the basic wind speed (m/s).

Step 6: Calculate the Projected Frontal Area

The projected frontal area of each section was determined using

Af= D * H

where (D) is the outside diameter and (H) is the section height.

The projected areas are:

- Sections 1–4 (M2): Af = 5.57

- Section 5 (M1): Af = 3.72

Step 7: Select the Force Coefficient

The force coefficient, (Cf), for the circular tubular pole was selected from the applicable design standard. Where antennas, ladders, cable ladders, and cables were present, their wind loads were considered as additional exposed components.

Step 8: Calculate the Wind Force

The equivalent static wind force acting on each mast section was calculated using

F = qz * G * Cf  * Af

where:

- (F) = wind force (N)

- (qz) = velocity pressure (Pa)

- (G) = gust effect factor

- (Cf) = force coefficient

- (Af) = projected frontal area (m²)

Step 9: Apply Wind Loads in the Finite Element Model

The calculated wind force for each section was applied as a lateral concentrated load at the centroid of the corresponding section in the ANSYS finite element model. The total wind loading on the monopole was obtained by summing the wind forces from all five sections.

Step 10: Perform Structural Analysis

The finite element analysis was carried out in ANSYS using the calculated wind loads. The structural response of the guyed tubular monopole, including deformation, stresses, support reactions, and member forces, was evaluated to assess the performance of the structure under the design wind loading.